DA20 modification to enable SW pass-through

(Cat. No. 611A card must be installed)

This modification will enable pin 24 (SubWoofer) of the Audio Input D-Connector to pass-through to pin 24 of the Audio Output D-Connector while the DA20 is either powered off or in “bypass” mode while playing through external audio.

Remove all of the screws around the perimeter of the back panel in order to remove the power supply and the D-Connector panel.

Label all of the ribbon cables with their associated mating “P” connector numbers for correct re-assembly.

Remove all of the ribbon cable connections to expose the DA20 backplane.

Locate the edge connector labeled “J19”. This edge connector contains three rows of 32 pins. Locate J19 pin A8.

Locate ribbon connector “P7”. This ribbon connector contains two rows of 13 pins. Locate P7 pin 24.

Carefully solder a jumper wire from P7 pin 24 to J19 pin A8. Double check to ensure no solder problems and that there are no stray strands to short circuit.

Re-assemble everything being careful to connect ribbon connectors appropriately.

The DA20 is now in full control!